# Overview

Datalogger là thiết bị thu thập và giám sát dữ liệu môi trường không khí, được thiết kế cho các ứng dụng quan trắc công nghiệp và đô thị thông minh. Sử dụng vi xử lý Arm® Cortex®-M7 hiệu năng cao, thiết bị đảm bảo khả năng xử lý nhanh, ổn định và đáp ứng tốt các tác vụ thời gian thực.

Hệ thống phần cứng được tích hợp đa dạng giao tiếp, bao gồm Ethernet 10/100 Mbps, LTE và tùy chọn WiFi, Zigbee hoặc LoRa, cho phép kết nối linh hoạt với các cảm biến và hạ tầng mạng khác nhau. Các cổng RS485, RS232 cùng hệ thống I/O (Digital Input/Output và Analog Input) giúp dễ dàng tích hợp với nhiều loại thiết bị đo và cảm biến môi trường. Ngoài ra, thiết bị được trang bị màn hình LCD cảm ứng 5 inch phục vụ giám sát trực tiếp, cùng khe cắm thẻ SD, bộ nhớ RAM lên đến 128 Mb và Flash lên đến 256 Mb để lưu trữ dữ liệu cục bộ. Nguồn cấp rộng 12~24 VDC phù hợp với nhiều điều kiện triển khai thực tế.

Về phần mềm, Datalogger hỗ trợ kết nối và thu thập dữ liệu từ các cảm biến chất lượng không khí xung quanh, thực hiện tính toán chỉ số chất lượng không khí (AQM) theo thời gian thực. Thiết bị có khả năng truyền dữ liệu đến server của Bộ Tài nguyên và Môi trường, đồng thời hỗ trợ giao thức MQTT/MQTTS để tích hợp với các nền tảng IoT. Cơ chế lưu trữ cục bộ đảm bảo không mất dữ liệu khi gián đoạn kết nối mạng và tự động đồng bộ lại khi kết nối được khôi phục. Ngoài ra, hệ thống hỗ trợ cập nhật firmware từ xa (FOTA), giúp dễ dàng nâng cấp và bảo trì thiết bị trong quá trình vận hành.

Với thiết kế linh hoạt, khả năng kết nối đa dạng và các tính năng phần mềm chuyên biệt cho quan trắc môi trường, Datalogger là giải pháp toàn diện cho các hệ thống giám sát chất lượng không khí hiện đại.

# Hardware Attributes

- Arm® Cortex®-M7

- 1 x Ethernet 10/100 Mbps

- 1 x LTE

- 1 x WIFI (Optional)

- 1x Zigbee or LoRa (Optional)

- 1 x USB

- 1 x SD card slots

- 2 x RS485

- 1 x RS232

- 1 x LCD touch 5 inch

- 8 x Digital Input

- 8 x Digital Output

- 8 x Analog Input

- up to 128 Mb Ram

- up to 256 Mb Flash

- 12~24 VDC input

# Software Attributes

# Software and tool

- STM32CubeMX

- STM32Programer

- CMake

- VSCode

- C/C++

# Blog Diagram

- sys.drawio

# Middleware

- FreeRTOS

- LWIP

- MbedTLS

- TinyUSB

- nanomodbus

- FATFS

- TouchGFX

- LitleFS

# Function

- Connect MQTT/MQTTS

- Connect FTP/FTPS

- Connect HTTP/HTTPS

- HTTP server configuration

- Caculator AQM

- Save data

- Control and Display with LCD

- FOTA (Secure Boot, Rollback Firmware, Update firmware over MQTT/HTTP

- Connect Zigbee sensor (or LoRa sensor) with ZigbeeStack (or LoRaWan)

- Modbus RTU over serial master and Modbus RTU over TCP server

1. SYSTEM OVERVIEW (Chuẩn hoá từ spec)

- Nguồn:

- # Datalogger System Overview

## Purpose
Datalogger is an embedded system for environmental monitoring (air quality), designed for:
- Industrial monitoring
- Smart city systems

## Core Capabilities
- Real-time data acquisition
- AQM calculation
- Multi-network connectivity
- Local storage + cloud sync
- Remote firmware update (FOTA)

## Connectivity
- Ethernet (Primary)
- WiFi (Optional)
- LTE (Backup)
- Zigbee / LoRa (Sensor network)

## Protocols
- MQTT / MQTTS
- HTTP / HTTPS
- FTP / FTPS
- Modbus RTU / TCP

## OS & Middleware
- FreeRTOS
- LWIP (TCP/IP stack)
- MbedTLS
- TinyUSB
- FATFS / LittleFS

- 📘 2. ETHERNET ARCHITECTURE

- # Ethernet Stack Architecture

## Thread Model
- App Thread
- LWIP Thread
- Ethernet IF Thread
- AT Process Thread (optional)

## Data Flow

App
 ↓
LWIP
 ↓
Ethernet Interface (NetIF)
 ↓
MAC Driver
 ↓
RMII Bus
 ↓
Ethernet PHY Module

## Components

### 1. Application Layer
- Uses BSD socket / MQTT / HTTP

### 2. LWIP
- TCP/IP stack
- Handles:
  - TCP
  - UDP
  - DHCP
  - DNS

### 3. Ethernet Interface (NetIF)
- Bridge between LWIP and driver
- Handles:
  - Packet input/output
  - IP binding

### 4. MAC Driver
- Low level Ethernet controller
- DMA + frame handling

### 5. Hardware
- RMII Bus
- External Ethernet PHY

- 📘 3. LTE (USB MODEM) ARCHITECTURE

- # LTE Modem Architecture (USB CDC)

## Note
Modem exposes 2 CDC ports:
- Port 0 → PPPOS (Data)
- Port 1 → AT Command (Control)

## Thread Model
- App Thread
- PPPOS Thread
- LWIP Thread
- AT Process Thread

## Data Flow

App
 ↓
LWIP
 ↓
PPPOS
 ↓
AT Port 0
 ↓
USB CDC Host
 ↓
LTE Modem

## Control Flow

App / AT Thread
 ↓
AT Port 1
 ↓
USB CDC Host
 ↓
LTE Modem

## Components

### 1. PPPOS
- PPP over Serial
- Converts serial → IP

### 2. AT Handler
- Send AT commands
- Parse modem response

### 3. USB CDC Host
- USB communication layer

### 4. LTE Modem
- Network provider connection

- 📘 4. WIFI (ST67W611M1) ARCHITECTURE

- # WiFi Module Architecture (AT-based)

## Interface
- SPI (data)
- UART (AT command optional)
- GPIO:
  - Device Ready
  - Chip Enable

## High Level Flow

App
 ↓
W6x APIs
 ↓
W6x Core
 ↓
AT Driver
 ↓
SPI Interface
 ↓
WiFi Module (ST67W611M1)

## Thread Model
- Application Thread
- AT Modem Process Thread
- SPI Sync Thread
- LWIP Thread
- Netif Thread

## Components

### 1. W6x Core
- Abstract WiFi stack
- Provide APIs to app

### 2. AT Driver
- Command-based communication

### 3. SPI Interface
- Data transport

### 4. LWIP
- TCP/IP stack

### 5. Netif
- Interface binding to LWIP

- 📘 5. NETWORK PRIORITY & FAILOVER

- # Network Failover Strategy

## Priority Order
1. Ethernet (High)
2. WiFi (Normal)
3. LTE (Low)

## State Machine

[Ethernet Connected]
    ↓ (Lost)
[Switch → WiFi]

[WiFi Connected]
    ↓ (Lost)
[Switch → LTE]

[LTE Connected]
    ↓ (Lost)
[Retry → Ethernet]

## Logic

IF Ethernet OK:
    Use Ethernet

ELSE IF WiFi OK:
    Use WiFi

ELSE:
    Use LTE

## Reconnect Strategy
- Always retry higher priority network

- 📘 6. FULL SOFTWARE ARCHITECTURE (Layered)

- # Software Architecture

## 1. Application Layer

- MQTT / MQTTS
- HTTP / HTTPS Client
- HTTP Server
- SNTP
- TCP / UDP
- Modbus TCP/IP
- LoRaWAN Gateway

## 2. Service Layer

- Network Services:
  - WiFi
  - BLE
  - LTE
  - LoRa

- System Services:
  - Logger
  - Storage
  - RS485
  - Command Line
  - AT Parser
  - Netif Manager

## 3. Hardware Abstraction Layer (HAL)

- GPIO
- UART
- SPI
- I2C
- USB Host/Device
- Ethernet

## 4. OS Layer

- FreeRTOS

## 5. Libraries

- LWIP
- TinyUSB
- MbedTLS
- FATFS
- LittleFS
- LoRaWAN Stack

- 📘 7. UNIFIED NETIF ARCHITECTURE (VERY IMPORTANT)

- 👉 Đây là phần quan trọng nhất để AI/code hiểu đúng system

- # Netif Abstraction Layer

## Goal
Unify all network types under ONE interface

## Interfaces

- netif_eth
- netif_wifi
- netif_lte

## Responsibilities

- Link up/down
- IP assignment
- Packet routing
- Failover switching

## API Design (Pseudo)

netif_init()
netif_set_active(type)
netif_get_active()
netif_is_connected(type)

## Integration with LWIP

Each netif registered into LWIP:
- netif_add()
- netif_set_default()

- 📘 8. THREAD ARCHITECTURE (FINAL NORMALIZED)

- # Thread Model

## Core Threads

- App Thread
- LWIP Thread
- Netif Thread

## Per Interface

### Ethernet
- Ethernet IF Thread

### WiFi
- SPI Sync Thread
- AT Process Thread

### LTE
- PPPOS Thread
- AT Process Thread

## Rules

- LWIP runs in single thread
- All network input → LWIP thread
- App NEVER access driver directly

- 📘 9. DATA FLOW (END-TO-END)

- # Data Flow

Sensor → MCU
    ↓
Application Logic
    ↓
Protocol Layer (MQTT/HTTP)
    ↓
LWIP
    ↓
Netif (eth/wifi/lte)
    ↓
Driver (MAC / PPP / SPI)
    ↓
Physical Interface
    ↓
Network